Exploring AI-Driven Construction Supervision for Human Activity

Frankie Ling Tai Yaw, Lu Min Yeu, Jee Sin Wee, Muhammad Ashrull bin Rahmad, and Lau Bee Theng

Swinburne University of Technology Sarawak Campus

# Current State and Challenges

Construction sites represent inherently dangerous environments where safety frequently hinges on a simple human factor: the neglect of Personal Protective Equipment (PPE). Workers often discard helmets or vests due to discomfort or mobility restrictions. Furthermore, traditional monitoring methods, which rely heavily on manual supervision, are often slow, inconsistent, and prone to error.

Deep learning detectors, particularly the You Only Look Once (YOLO) architecture, have become the industry standard for PPE recognition. Research by Akmal, Lestari, and Pradana [1] demonstrated that these models can achieve accuracy rates exceeding 95% in controlled environments. However, real-world construction sites are rarely controlled. In this case, construction sites often consist of chaotic environments plagued by dynamic lighting, dust, and visual obstructions, which frequently cause standard models to fail.

The failure is largely due to standard object detection, which suffers from a lack of temporal and spatial reasoning. Identifying that a worker is present is insufficient; the model must also understand if the worker is in a hazardous state. Abbas et al. [2] highlighted that without temporal logic, such as tracking an unsafe activity over time, the automated hazard detection systems trigger excessive false alarms, leading operators to ignore alerts. More recent approaches by Gil and Lee [3] have attempted to address this by using image captioning for zero-shot monitoring, shifting the focus from simple bounding boxes to scene description. According to the review, there is a need for a mobile workplace safety monitor to be placed at construction sites that goes beyond simple object detection, but can understand the specific construction contexts. With the advancement of technologies, this can be explored and prototyped by integrating discriminative computer vision with GenAI to identify missing PPE, while also comprehending the significance of the violation, and generating real-time risk reports that adhere to national and international safety guidelines.

# Proposed Solution

To address the identified gap, the project proposed a hybrid model that builds upon the image captioning technology by extending it with a Retrieval-Augmented Generation (RAG) pipeline. This ensures that every detected risk is not only described in natural language but is also cross-referenced against the official Guidelines for Public Safety and Health at Construction Sites (1994) from the Department of Occupational Safety and Health (DOSH) [4].

A local pipeline was designed to ensure data privacy and minimise latency, a critical requirement for future deployment of mobile robotics. The system operates through four distinct, interconnected stages.

For computer vision-based detection, the proposed system utilises the YOLOv8 architecture. Initially, Faster R-CNN was employed. However, it proved that the approach is too computationally intensive for the available hardware. This architecture offered a superior balance of speed and accuracy, achieving high detection rates for core items, such as helmets and vests, while remaining efficient enough for edge devices.

To establish context, vision-language models are adopted. The system integrates LLaVA 1.5-7b-hf. This model acts as a bridge between visual data and reasoning by generating descriptive captions for the scene. It identifies the number of workers and their specific activities. For instance, it can differentiate between a worker standing still and one bending down to cut concrete blocks. This textual data provides a depth of risk understanding that simple bounding boxes cannot achieve.

In the safety-critical construction contexts, the possible

GenAI hallucination is risky. To prevent this, a Retrieval-Augmented Generation pipeline was implemented using ChromaDB. Rather than relying on generic safety data, the system embeds the official DOSH guidelines [4]. When a hazard is detected, the system retrieves the precise legal regulation that applies, ensuring all advice adheres to the national and international guidelines.

Locally hosted large language model analysis, the Llama 3 model is adapted to process scene data. It synthesises the scene description, the YOLO detection results, and the retrieved DOSH regulations to generate a structured risk report. While DeepSeek R1 was used initially, the system was eventually migrated to Llama 3 to facilitate the generation of a standardised JSON report, which includes severity levels and specific action items.

# IV. Development

Significant effort was dedicated to dataset preparation, including OSHA records and Kaggle data, which were consolidated into a single training set. This process involved normalising inconsistent label names and balancing class distributions. Additionally, the 500 construction scene images were compiled to ensure robust model learning.

DATASET SAMPLES

In addition to detection, the system generates comprehensive reports. A web-based dashboard was developed for safety officers, featuring a live video feed with bounding boxes highlighting hazards. When a violation occurs, the system logs the event and generates a PDF report. This report cites specific DOSH regulations, such as Section 21.5.1 regarding safety helmets. Furthermore, the dashboard categorises hazards by severity, enabling officers to prioritise interventions effectively.

USER INTERFACE

# V. Discussions

The prototype was evaluated with both YOLOv8 and YOLOv11, demonstrating robust performance. The YOLOv8 model achieved strong metrics, recording a mean Average Precision (mAP@50) of 82.8% for helmets, 82.5% for vests, and 81.3% for safety boots.

Table 1

Results

| Class | YOLOv8 mAP50 | YOLOv8 mAP50-95 | YOLOv11 mAP50 | YOLOv11 mAP50-95 |
| --- | --- | --- | --- | --- |
| Helmet | 0.8287 | 0.4531 | 0.8223 | 0.4360 |
| Gloves | 0.8047 | 0.3855 | 0.7940 | 0.3906 |
| Vest | 0.8255 | 0.5239 | 0.8512 | 0.5274 |
| Boots | 0.8132 | 0.4681 | 0.7982 | 0.4639 |
| Goggles | 0.7873 | 0.3861 | 0.7871 | 0.3871 |

# VI. Future Works

In the next phase, the project will transition to field deployment. The system will be implemented on mobile robots which can perform real-time construction site supervision. This integration will enable the system to autonomously patrol sites, detecting work activities and risks, thereby transforming the role of human construction supervisors.

The backend infrastructure will be fully cloud-based, and the local database will be migrated to a cloud-based Supabase. This migration will facilitate long-term data retention, allowing for the analysis of safety trends over time. Further optimisation will be conducted to enhance detection precision and improve inference speeds when running on the mobile robot's onboard hardware.

VII.

References

- B. K. Akmal, W. Lestari, and A. I. Pradana, "Personal protective equipment completeness monitoring system using YOLO-based computer vision," Journal of Applied Informatics and Computing, vol. 9, no. 4, 2023.

- A. Abbas, F. Xu, S. Rahman, and Kim, "Temporal logic-based mobile monitoring for construction safety management," Automation in Construction, vol. 161, p. 105069, 2025.

- D. Gil and G. Lee, "Zero-shot monitoring of construction workers personal protective equipment based on image captioning," Automation in Construction, vol. 164, 2024.

- Department of Occupational Safety and Health (DOSH), "Guidelines for Public Safety and Health at Construction Sites," 1994.